[NOTE: ALL HANDWRITTEN WORK IS AT END OF FILE. FILLED IN ALL ANSWERS IN WORD FOR EASE OF READING AS I HAVE PRETTY TERRIBLE HANDWRITING]

# Q1. Cache and Memory mapping (6 points)

Suppose a byte-addressable memory has 2M byte capacity and cache consists of 64 blocks, where each block contains 32 bytes.

1.    Direct Mapping

1) Divide the bits in main memory into tag, block and offset bits.

| TAG | ROW/LINE | OFFSET |
| --- | --- | --- |
| 10 | 6 | 5 |

2) What is the tag, line and offset for the address $123A63, in hexadecimal?

Tag:       0x247

Line:      0x13

Offset:   0x03

2.   Fully associative mapping

1) Divide the bits in main memory into tag and offset bits.

| TAG | OFFSET |
| --- | --- |
| 16 | 5 |

2) What is the tag and offset for the address $123A63, in hexadecimal?

Tag:       0x91D3

Offset:   0x03

3.    4-way set associative mapping

1) Divide the bits in main memory into tag, set and offset bits

| TAG | SET | OFFSET |
| --- | --- | --- |
| 12 | 4 | 5 |

2) What is the tag, set and offset for the address $123A63, in hexadecimal?

Tag:       0x91D

Set:       0x3

Offset:   0x03

# Q2. Cache hit and miss (3 points) Slide 60

Suppose we have a computer that uses a memory with 256 byte capacity. The computer has a 16-byte direct-mapped cache with 4 bytes per block. The computer accesses a number of memory locations throughout the course of running a program. Here is the memory addresses in this exact order: 0x91, 0xA8, 0xA9, 0xAB, 0xAD, 0x93, 0x6E, 0xB9, 0x17, 0xE2, 0x4E, 0x4F, 0x50, and 0xA4. The cache is already filled out as shown below. (The contents of the tag are shown in binary and the cache “contents” are simply the address stored at that cache location)

| Tag | Block # | offset 0 | offset 1 | offset 2 | offset 3 |
| --- | --- | --- | --- | --- | --- |
| 1110 | 0 | E0 | E1 | E2 | E3 |
| 0001 | 1 | 14 | 15 | 16 | 17 |
| 1011 | 2 | B8 | B9 | BA | BB |
| 0110 | 3 | 6C | 6D | 6E | 6F |

1. What is the hit ratio for the entire memory reference sequence given (in bold)?

14 Total Address. 5 hit, 9 miss. 35.7% Hit Ratio

2. What memory will be in the cache after the last address has been assessed?

| Tag (binary) | Block # | offset 0 | offset 1 | offset 2 | offset 3 |
| --- | --- | --- | --- | --- | --- |
| 0101 | 0 | 50 | 91 | E2 | 93 |
| 1010 | 1 | A4 | 15 | 16 | 17 |
| 1011 | 2 | A8 | B9 | BA | AB |
| 0100 | 3 | 6C | AD | 4E | 4F |

| ADDRESS | TAG | BLOCK | OFFSET | HIT/MISS |
| --- | --- | --- | --- | --- |
| 91 | 1001 | 00 | 01 | MISS |
| A8 | 1010 | 10 | 00 | MISS |
| A9 | 1010 | 10 | 01 | HIT |
| AB | 1010 | 10 | 11 | HIT |
| AD | 1010 | 11 | 01 | MISS |
| 93 | 1001 | 00 | 11 | HIT |
| 6E | 0110 | 11 | 10 | MISS |
| B9 | 1011 | 10 | 01 | MISS |
| 17 | 0001 | 01 | 11 | HIT |
| E2 | 1110 | 00 | 10 | MISS |
| 4E | 0100 | 11 | 10 | MISS |
| 4F | 0100 | 11 | 11 | HIT |
| 50 | 0101 | 00 | 00 | MISS |
| A4 | 1010 | 01 | 00 | MISS |

# Q3.  Virtual memory and cache (6 points)

Consider a processor with the following memory hierarchy:

- 256K virtual address space (byte addressable)

- 128K physical address space, each page (frame) has 32K bytes (byte addressable)

- 2Kbyte direct-mapped cache, a block (refill line) has 256 bytes

- The machine uses a two entry TLB.

- All replacement policies are LRU. There are two LRU stack.

- TLB LRU is used whenever an entry of TLB is replaced.

- Mem LRU is used when a mapping is replaced when page fault happens.

- The entry of these stacks are the page number of a virtual memory.

- Note that all the values are represented as hexadecimal.

TLB

| Virtual page # | Physical page # | Valid |
| --- | --- | --- |
| 5 | 3 | 1 |
| 0 | 2 | 1 |

TLB LRU stack

0

5

Page Table

| Virtual page # | Physical page # | Valid |
| --- | --- | --- |
| 0 | 2 | 1 |
| 1 | 1 | 1 |
| 2 | --- | 0 |
| 3 | --- | 0 |
| 4 | 0 | 1 |
| 5 | 3 | 1 |
| 6 | --- | 0 |
| 7 | --- | 0 |

Mem LRU stack

0

5

4

1

Cache

| Line # | Tag | Data block |
| --- | --- | --- |
| 0 | 10 | * |
| 1 | 0A | * |
| 2 | 3C | * |
| 3 | 14 | * |
| 4 | 28 | * |
| 5 | 04 | * |
| 6 | 37 | * |
| 7 | 1D | * |

1. Split the bits of virtual address and physical address

| Virtual | Page # | Offset |
| --- | --- | --- |
|  | 3 | 15 |
| Physical | Frame # | Offset |
|  | 2 | 15 |

2. Split the bits in memory address based on the cache

| TAG | ROW/LINE | OFFSET |
| --- | --- | --- |
| 6 | 3 | 8 |

3. Suppose the processor has requested to access a memory in 0x32764 (which is virtual address)

1) Is it a page fault? Explain.

Yes, page fault occurs, 6 was not in TLB (miss) so went to page table for lookup. 6 was not mapped so PAGE_FAULT occurred

2) Show the changes of TLB, TLB LRU, page table and Mem LRU

| TLB | Virtual page # | Physical page # | Valid |
| --- | --- | --- | --- |
|  | 6 | 1 | 1 |
|  | 0 | 2 | 1 |

TLB LRU stack

6

0

Page Table

| Virtual page # | Physical page # | Valid |
| --- | --- | --- |
| 0 | 2 | 1 |
| 1 | --- | 0 |
| 2 | --- | 0 |
| 3 | --- | 0 |
| 4 | 0 | 1 |
| 5 | 3 | 1 |
| 6 | 1 | 1 |
| 7 | --- | 0 |

Mem LRU stack

6

0

5

4

3) Show the changes in Cache. 0x32764(Virtual) -> 0x0A764

| 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TAG |  |  |  |  |  | ROW/LINE |  |  | OFFSET |  |  |  |  |  |  |  |
| 0x14 |  |  |  |  |  | 7 |  |  | b |  |  |  |  |  |  |  |

Cache

| Line # | Tag | Data |
| --- | --- | --- |
| 0 | 10 | * |
| 1 | 0A | * |
| 2 | 3C | * |
| 3 | 14 | * |
| 4 | 28 | * |
| 5 | 04 | * |
| 6 | 37 | * |
| 7 | 14 | * |